- Leica DM750 Compound Binocular Microscope

Microscope Features and Specifications:

Optical System

- Universal Infinity System with HCS (Harmonic Component System) concert indicating that all the performance parameters-objectives, tubes lenses, tubes, eyepieces, TV/camera adapters and etc. have been harmoniously balanced throughout the entire optic system, allowing to create more scope for even more technical capability

- Allowing attachment to be added between observation tube and objective without any magnification factor change.

- Permitting observation of various contrast technique with a single set of objective with highest observation method.

- With Chromatic Aberration fully corrected type.

- Allowing up to two intermediate attachments to be attached at the same time.

- Microscope with advance optical system to enable to perform contrasting method of other application such as Phase contrast, Dark field, Polarization contrast, Photomicrography and etc.

Frame

- Ergonomic design allows operator to move arm in a natural, comfortable manner during operation.

- Stage movement control and focusing knobs are easily reachable. Thus, eliminate operator from having to raise arms and wrist from the work surface.

- Die cast aluminum stand to minimize vibrations for durability, stability and fatigue-free operation.

- Integrated vertical handle provides easy carrying and easy lifting when storing on high shelves; undercut on front of stand works in combination with the handle for safer, two-handed carrying

- AGTreat™ All microscope touchpoints are treated with an additive to inhibit the growth of bacteria. This helps to prevent the spread of disease via the microscope surfaces and leads to a healthier laboratory environment

EZStore™

- Integrated cord wrap eliminates damage to microscope components from improper cord wrapping; vertical cord insertion prevents the cord from pulling partially out of the stand while in storage or in use

- Unique shape of the microscope stand protects controls from damage when microscopes are stored side-by-side

Focus

- Weighted focus knobs provide inertia and extremely accurate focus capability

- Focus mechanism with self-adjusting and maintenance-free focus mechanism

- One full coarse focus turn equals 2.8mm.

- One full fine focus turn equals 300 microns.

- Fine knob scale: Calibrated in 3 micron increments

Illumination - EZLite™

- LED illumination 6000 K temperature with 25,000 hour life at full intensity, provides cool, white light

- Lifetime of over 20 years of average use. No longer need to change lamps during lab time and save the expense of replacement lamps

- LED illumination designed to provide even lighting across the full field of view without any adjustments

- Light intensity adjustment knob

- Preset Field aperture

- Patented time delay shutoff saves energy by automatically turning off the illumination after 2 hours of no use

- Koehler field diaphragm available as an option for optimum illumination and contrast

Revolving Nosepiece

- Rear/inward facing quadruple (4-fold) nosepiece that allows easy access to the sample and prevents colliding with objectives with rubber grip on turret.

- Can be upgrade to quintuple (5-fold) nosepiece

Observation Tube

- Binocular eyepiece tube with 30° viewing angle

- Continuous interpupilary distance adjustment from 52 - 75mm without the need for refocusing.

- Anti-fungus/mold treated type.

- Patented captive thumbscrew for safe rotation of the tube provide free rotation while tube securely fixed to the stand for easy sharing and storage.

Eyepiece

- Pair of 10x wide field eyepiece with an efficient field number of 20mm, with foldable eyeguards

- With eyepiece locking screws to prevent loss of eyepieces

- One piece with diopter adjustment

- Anti-fungus/mold treated type

Mechanical Stage - SafeTStage™

- Microscope stage maintains its dimension, which eliminates the chance of injury from contact with a conventional stage rack

- Rounded edges are easy on the skin

- Wear resistant stage surface

- Stage size: 150 x 140mm for easy recording of coordinates with vernier scales.

- Traversing Y-axis of 26mm.

- Traversing X-axis of 76mm.

- Vernier scale of 1mm on main scale.

- Coaxial low drive with right hand mounted.

- EZGuide™ with student friendly slide holder to prevent slide chipping.

- One-hand loading specimen holder

Condenser

- Standard Abbe Condenser 0.90 Dry / 1.25 Oil, centerable and focusable condenser mount.

- Corrected for spherical and chromatic abberation

- With slot in the condenser for contrast sliders (Phase, Darkfield, Compensator)

- Aperture iris diaphragm has an objective magnification scale (4x, 10x, 40x, 100x) resulting in high resolution and high contrast image.

Objective

- Plan Achromatic corrected objective, support Brightfield, Darkfield and Polarisation techniques with DIN colour coded and marked magnifications of:

  - Plan 4x/0.10NA, 26.2mm W.D.

  - Plan 10x/0.22 NA, 7.8mm W.D.

  - Plan 40x/0.65 NA, 0.31mm W.D. Spring loaded.

  - Plan 100x/1.25 NA, 0.10mm W.D., Oil. Spring loaded.

- All objectives are Parfocal and Precentered to minimize refocusing

Certifications

- Comply with cULus, CE and RoHS regulations.

Complete microscope delivery including:

13613510	Leica DM750 Basic Stand Mechanical Stage RH

13613521	30° Binocular Tube

13613530	Eyepiece 10x/20mm

13613532	Eyepiece 10x/20mm, focusable

13613550	Abbe condenser 0.90 Dry / 1.25 Oil

13613240	Plan 4x/0.10NA, 26.2mm W.D.

13613241	Plan 10x/0.22 NA, 7.8mm W.D.

13613242	Plan 40x/0.65 NA, 0.31mm W.D., spring-loaded

13613243	Plan 100x/1.25 NA, 0.10mm W.D., Oil, spring-loaded

13613902	Power cord

13614800	Immersion oil, 10ml

13613584	Dust cover