RED INSULATING VARNISH (GC-88)

DESCRIPTION

Red Insulating Varnish is a fast drying, red insulating enamel characterized by out standing chemical resistance properties. This enamel provides a smooth, glossy surface, which sheds dirt readily. Especially recommended for coating end turns on electrical Windings that will be exposed to extreme moisture and corrosion condition.

APPLICATIONS

Air-dries to a hard, glossy flexible state, providing good adhesion and high dielectric strength. Has excellent resistance to moisture and oils. Has superb resistance to acid, alkalis and chemicals. Air-dries tack-free in 15 minutes. A proven insulator with high-arc resistance, recommended for Motor windings Rings and frames

Commutator ends Transformers Field coils Switch board parts Armatures Sealing electrical/electronic components Stator Windings Bus bars

SPECIFICATIONS

| Type | Red Insulating Enamel |
| --- | --- |
| Thinner | Xylol |
| Color | Red |
| Air Drying Time on Copper (ASTM) | Tack free 15 minutes |
|  | Hard 1 hour |
| Oil Resistance (ASTM) | Satisfactory |
| Adhesion to Metal | Excellent |
| Alkalis Resistance | Excellent |
| Moisture Resistance | Excellent |
| Acid Resistance | Excellent |
| Salt Spray Resistance | Excellent |
| Dielectric Strength | 2400 VPM – Dry 1200 VPM - Wet |